User Requirements Specification

R24 Continuous Hydrogenation

System ID: 902/198

Approvals

Completion of the following signature section signifies the approver(s) has read, understands, and authorises the use of this User Requirements Specification for the above-mentioned System.

| Prepared by: | Keith Jefferies |  |
| --- | --- | --- |
| Process and Project Engineer: | (Printed Name & Signature) | Date (DD-MMM-YYYY) |
| System Owner: | Barry Goligher |  |
| Operations Lead: | (Printed Name & Signature) | Date (DD-MMM-YYYY) |
| Facility Lead: | Scott Anderson |  |
| Executive Director | (Printed Name & Signature) | Date (DD-MMM-YYYY) |
|  | Steve Last |  |
| Quality Assurance | (Printed Name & Signature) | Date (DD-MMM-YYYY) |

For electronic signature - enter “Electronically Signed” above.

Contents

1.	Objective	4

2.	Scope	4

3.	Roles and Responsibilities	4

4.	System Description	4

5.	System Life Cycle	7

6.	Constraints	8

7.	Equipment Operational Requirements	9

8.	Equipment Construction Requirements	10

9.	Installation Requirements	12

10.	Equipment and Computerised System Interfaces	14

11.	Computerised System Functional Requirements	14

12.	Computerised System Data Requirements	15

13.	Data Integrity Gap Analysis Requirements	16

14.	Asymchem Sandwich Documentation Requirements	16

15.	Supplier Documentation Requirements	16

16.	System Support and Training Requirements	17

17.	Document History	17

# Objective

The purpose of this User Requirements Specification (URS) is to clearly describe the business and regulatory requirements for the R24 Continuous Hydrogenation equipment for use in B902 Pilot Plant, Sandwich and provide a clear, unambiguous description of what the continuous hydrogenation process must do, with sufficient technical information to enable a supplier to provide a tender/quotation for the system.

The URS defines what the user wants the system to do, including the functions to be carried out, the data on which the system will operate, and the operating environment. This URS also defines any non-functional requirements which apply to the system.

# Scope

The scope of this document is to provide the requirements and system specifications for the design, fabrication, assembly, testing, delivery, installation, commissioning and qualification of the new R24 Continuous Hydrogenation process to be installed in Pilot Plant at Asymchem Ltd, Sandwich.

# Roles and Responsibilities

The roles and responsibilities for the project are managed via a Quality Task Matrix (QTM).

# System Description

The R24 Continuous Hydrogenation System consists of a proprietary Asymchem 3-stage plug flow hydrogenation reactor skid, capable of producing nominally 30kg/day of Active Pharmaceutical Ingredient (API)/Intermediate material.

The system will provide nominally 8 hours of buffer capacity for raw materials, product collection and hydrogen gas.

Catalyst will be pre-dispensed into contained charge containers/bags in the Pilot Plant IPC laboratory fume hoods and charged to the reactor columns via a contained catalyst charging system.  Spent catalyst will be washed with water prior to discharging to sealed containers.

R24 will share the R23 hydrogen service located adjacent to the pilot plant, and raw materials will be introduced into the buffer tank via either fixed transfer lines, mobile head tanks or drums.  Product will be transferred from the product collection tank to either mobile head tanks or drums for further processing.

The R24 reactor system will be inerted prior to introduction of any flammable materials and the process/emergency vent headers will be nitrogen purged during normal operation.

Hydrogen is introduced into the reactor system at a controlled flowrate of between 0.1 and 1 std m3/hr at a pressure up to 60barg and the reactor system pressure will be controlled based on the separator pressure.

Raw materials from the feed tank will be introduced into the reactor system at a controlled flowrate of between 2 and 20ltr/hr at a pressure up to 60barg.

Prior to feeding the liquid and gas streams into the plug flow reactors, the gas/liquid streams are heated/cooled in the pre-heater to a temperature between -20 and +130°C.  The gas/liquid streams enter the reactor at the base and pass through a distribution plate before flowing through a 1.5m x 80mm catalyst bed at a velocity of up to 2.5mm/s.  Temperature in the plug flow reactor is controlled at a pre-set temperature between -20 and +130°C.  A further two 1.5m x 65mm diameter reactors, each with independent temperature control, are available within the system, allowing either stepwise adjustment of temperatures within the systems -20 to +130°C design range, or increased residence time, at a velocity of up to 3.5mm/s, to ensure the hydrogenation reaction runs to completion.

Following the reaction step, the gas/liquid stream is cooled via an inline condenser, and then unreacted hydrogen gas is separated from the liquid phase via a separator.  The vapour phase from the separator is vented directly to atmosphere via a back pressure control valve, which controls the system pressure, and the liquid phase is discharged via a pressure reducing valve and modulating control valve, to control liquid level in the separator, to the atmospheric product collection tank.

Overpressure protection of the reactor system will be incorporated into the design, and emergency venting of process relief streams will be routed to R23-CT02 catch tank to prevent a loss of containment of active/intermediate material during two-phase relief.

Control of the continuous hydrogenation system is via a Siemens S7 PLC and batch cards are used to manage the hydrogenation process.  The system will be controlled locally via either a local HMI positioned on the skid package, or via a panel mounted HMI in the R23 local control room.  There will be a hardwired control interface to the existing DeltaV PCS for control and safety interlocking of the hydrogen service to R23 module, and a comms link to enable remote monitoring of R24 status and provide historical trending of process parameters.

Cleaning of the reactor system is via a manually controlled CIP pump system, with spray balls providing 100% coverage of product contact surfaces within the feed tank, product collection tank and separator.  Due to the small volume of the system, cleaning of the reactors, interconnecting pipework and buffer/product tanks will be carried out on a once through basis.  Release of the system following a campaign clean down, will be via a combination of chemical analysis of rinse solutions, visual inspection of feed and product vessel internals (via sight glasses), and calculation of residual hold up in small-bore vent lines and the separator (to confirm potential holdup has negligible impact on subsequent processing).

The continuous hydrogenation system will be supplied from Asymchem China and integrated into the R23 process module at Asymchem Sandwich.  The design of R24 will enable normal operation of either R23 or R24, without impacting the operation of the other hydrogenator.

# System Life Cycle

| Req. No. | Requirement | Category |
| --- | --- | --- |
|  | ## System Testing Requirements: The Asymchem Sandwich System Life Cycle shall be followed as described in SOP-VAL-001 Equipment & Computerized Validation Systems Life Cycle (SLC) Requirements. | GMP |
|  | ## System / Supplier Quality System Requirements: The Asymchem Sandwich GMP Impact and Risk Assessment review will be performed to determine the qualification work required, prior to the use of this system for GMP manufacture. | GMP |
|  | ## System / Supplier Quality System Requirements: Change Control/QTM will focus validation efforts and document deliverables supporting this project. | GMP |
|  | ## System / Supplier Quality System Requirements: KU-0131 will be reviewed and updated to incorporate the introduction of R24 hydrogen service tie-in. | GMP |
|  | ## System / Supplier Quality System Requirements: A URS for R24 PLC control system will be developed to detail the automation requirements for the continuous hydrogenation system, including interfaces to the DeltaV PCS and standalone TCU controllers. | GMP |
|  | ## System Testing Requirements: Commissioning, testing and verification of the new continuous hydrogenation system will be done through Installation & Operational Qualification (IQ/OQ) and Functional Safety Validation Protocols. | GMP, EHS |
|  | ## System Testing Requirements: Occupational Hygiene monitoring of catalyst dispensing, charging and discharging operations is required to demonstrate OEB 4 containment has been achieved. | GMP, EHS |
|  | ## System Testing Requirements: Any supplier commissioning documentation that is intended to be used as part of the Asymchem Sandwich equipment validation, must be pre-approved by Asymchem Sandwich Quality Assurance (QA) | GMP |
|  | ## EHS System Requirements: EHS pre-project checklist will be completed to determine the EHS system requirements for this project. | EHS |
|  | ## EHS System Requirements: An update to the facilities functional safety (FS) documentation will be required. If necessary, a project specific functional safety management plan will be developed to define the high-level FS strategy to be adopted. | EHS |
|  | ## EHS System Requirements: Engineering documentation and schedules (PFD’s, P&ID’s, Equipment Schedules, Line Schedules, Valve Schedules, Instrument Schedules, SP Schedules) will be developed/updated to support introduction of this capability to Asymchem, Sandwich. | EHS |
|  | ## EHS System Requirements: Overpressure protection of equipment must be considered during detailed design and pressure relief studies carried out to document safety critical parameters/instruments/equipment. | EHS |
|  | ## EHS System Requirements: Safety Critical Equipment and Safety Critical Instrument Schedules will be updated to document safety critical design aspects for this project. | EHS |

# Constraints

| Req. No. | Requirement | Category |
| --- | --- | --- |
|  | Regulatory / Compliance Requirements: The system shall be designed and assembled to provide good drainability and promote cleanability in accordance with GMPs for Pharmaceutical Process Equipment. | GMP |
|  | Regulatory / Compliance Requirements: The system shall be designed to contain OEB 4 materials without the need for primary PPE to protect operating personnel. | EHS |
|  | Regulatory / Compliance Requirements: Calibration of all critical instruments shall be carried out by qualified personnel / appointed subcontractors. | GMP |
|  | Regulatory / Compliance Requirements: Asymchem Sandwich instrument calibration and categorisation procedures shall be followed to capture the calibration requirements of all GMP/Safety critical instrumentation. | GMP, EHS |
|  | Regulatory / Compliance Requirements: The emissions to atmosphere from the continuous hydrogenation plant must be less than 2kg/hr for Class B solvents and less than 0.1kg/hr for Class A solvents. | EHS |
|  | Statutory, Safety and Legal Requirements: All equipment shall at a minimum comply with the Supply of Machinery (Safety) Regulations 2008 / Machinery Directive 2006/42/EC. | EHS |
|  | Statutory, Safety and Legal Requirements: All low voltage electrical equipment shall at a minimum comply with the Electrical Equipment (Safety) Regulations 2016 / Low Voltage Directive 2014/35/EU. | EHS |
|  | Statutory, Safety and Legal Requirements: All equipment shall at a minimum comply with the Electromagnetic Compatibility Regulations 2016 / EMC Directive 2014/30/EU. | EHS |
|  | Statutory, Safety and Legal Requirements: The system shall comply with the UK Pressure Equipment (Safety) Regulations 2016 / Pressure Equipment Directive 2014/68/EU. | EHS |
|  | Statutory, Safety and Legal Requirements: The system shall comply with the UKEX - The Equipment and Protective Systems Intended for Use in Potentially Explosive Atmospheres Regulations 2016 / ATEX Directive 2014/34/EU. | EHS |
|  | Statutory, Safety and Legal Requirements: The continuous hydrogenation system must be CE/UKCA marked. | EHS |
|  | Statutory, Safety and Legal Requirements: The R23 module is designated as a hazardous area. The default zone for the module is ATEX Zone 2 IIC T4 unless additional, specific bubble zoning requirements have been designated. | EHS |
|  | Statutory, Safety and Legal Requirements: The system shall be designed and provided with suitable grounding for all components and provide sufficient earth continuity to a single external grounding connection. | EHS |
|  | Statutory, Safety and Legal Requirements: The system shall be provided with all of the safety features required for the protection of operating and maintenance personnel. | EHS |
|  | Statutory, Safety and Legal Requirements: Machinery and accessories for lifting loads must be clearly marked to indicate safe working loads and comply with The Lifting Operations and Lifting Equipment Regulations 1998. | EHS |
|  | Statutory, Safety and Legal Requirements: All system nameplates must include Asymchem Sandwich tag numbers (where applicable), temperature and pressure rating (design and working value if applicable), material of construction, CE/UKCA marking, ATEX rating and relevant manufacturer information. | EHS, BUS |
|  | System Availability/ Reliability Requirements: The system should be available for use on a 24 hour, 7 days a week basis. | BUS |

# Equipment Operational Requirements

| Req. No. | Requirement | Category |
| --- | --- | --- |
|  | Equipment Functions: The continuous hydrogenation system shall be capable of producing nominally 30kg/day of API/Intermediate. Note: a minimum rate cannot be specified as this is dependent on API/Intermediate chemistry which is not available at this time. | BUS |
|  | Equipment Functions: The continuous hydrogenator will consist of 2 no. 65mm diameter x 1.5m reactor columns and 1 no. 80mm diameter x 1.5m column. Each column will have a nominal catalyst bed height of 975mm. | BUS |
|  | Equipment Functions: Catalyst will be charged to the reactors via an OEB 4 contained charging system. | EHS |
|  | Equipment Functions: The continuous hydrogenation system will be pressure tested prior to the introduction of hydrogen gas. A leak test to 0.5barg will precede the pressure test and can be carried out local to the skid. The pressure test will be carried out remotely, and the test pressure will be a minimum of 110% of the equipment operating pressure i.e. for a 10barg process the pressure test will be carried out at a minimum 11barg. A leak rate of less than 200mbar over a 15-minute test period must be achieved, prior to introduction of hydrogen. | EHS |
|  | Equipment Functions: The continuous hydrogenation system, including feed and product collection tanks, will be inerted with nitrogen prior to the introduction of flammable materials. | EHS |
|  | Equipment Functions: The continuous hydrogenation system will be capable of establishing a flow of hydrogen gas between 0.1 and 1.0 std m3/hr ± 0.1 std m3/hr, at pressures between 10 and 60 barg. | BUS |
|  | Equipment Functions: The continuous hydrogenation system will be capable of controlling system pressure (based on the separator pressure) from 10 to 60 barg ± 2 barg. | BUS |
|  | Equipment Functions: The liquid feed to the continuous hydrogenation system will comprise of one buffer tank capable of holding a minimum 8 hours of feed material. | BUS |
|  | Equipment Functions: The liquid feeds will be metered into the system at a flowrate of between 2 and 20 ltrs/hr against a back pressure of up to 60 barg. | GMP, BUS |
|  | Equipment Functions: The temperature of the liquid/gas feeds to the reactors will be controlled within a temperature range of -20 to 130°C ± 2°C (based on jacket temperature). | GMP, BUS |
|  | Equipment Functions: The temperature of the reaction mix in the 3 continuous reactors will be independently controlled and within a temperature range of -20 to 130°C ± 2°C (based on jacket temperature). | GMP, BUS |
|  | Equipment Functions: The temperature of the condenser will be controlled within a temperature range of 10 to 30°C ± 2°C (based on jacket temperature). | EHS, BUS |
|  | Equipment Functions: The design temperature of the system will be -25 to 150°C. | EHS |
|  | Equipment Functions: The design pressure of the continuous hydrogenation system will be -1 to 80 barg. | EHS |
|  | Equipment Modes of Operation: The DeltaV PCS will control routing of the hydrogen service to the R24 continuous hydrogenation system. Hydrogen detectors will be located in the vicinity of the system to detect leaks during routine operation and automated safety shutdown sequences will be developed to disable the hydrogen service on detection of a leak. | EHS |
|  | Equipment Modes of Operation: The continuous hydrogenation system will be controlled via a Siemens S7 PLC. Sequences will be developed for inertion, hydrogen feed, liquid feed, product transfer and system depressurisation. Cleaning of the system will be sequenced via an automated sequence. | EHS, BUS |
|  | Equipment Modes of Operation: Lamps and Indicators will display equipment modes / status. As a minimum, critical temperatures and pressures will be clearly displayed to the operator. | BUS |

# Equipment Construction Requirements

| Req. No. | Requirement | Category |
| --- | --- | --- |
|  | Materials of Construction: Process wetted (product contact) parts shall be Stainless Steel, Type 316L. | GMP |
|  | Materials of Construction: Non-wetted (non-product contact) parts shall be 304 Stainless Steel (or approved alternatives). | BUS |
|  | Materials of Construction: The continuous hydrogenation skid framework shall be made of Stainless Steel (304 - minimum standards). | BUS |
|  | Materials of Construction: Process wetted gaskets shall be PTFE, PTFE encapsulated, Kalrez or perfluoroelastomer. | GMP |
|  | Materials of Construction: The equipment should not include any threaded connections on process pipe-work or product contact areas unless specifically allowed for in the relevant piping specifications. | BUS |
|  | Materials of Construction: Process wetted (product contact) parts shall be polished to a minimum surface finish of less than 0.5µm Ra (measured to BS1134 2010 or equivalent). | GMP |
|  | Dimensional Limitations: Feed/discharge chutes will be designed with a minimum 60° angle to aid powder/catalyst flow. | BUS |
|  | Lubrication: Any fluids or lubricants likely to come into contact with product, shall be approved for pharmaceutical use, e.g., USDA Food Grade or Pharmaceutical Grade, and shall have BSE/TSE certification | GMP |
|  | Dimensional Limitations: Loading positions shall be no higher than 1.2m above floor level. If loading positions cannot meet this requirement suitable access platforms must be provided. | EHS |
|  | Dimensional Limitations: The system components must be able to be transported to the Pilot Plant 3rd Floor area via the goods lift. The lift car allows for a 2.5 m high by 2.0 m wide x 2.1m deep component, weighing less than 2,000kgs, to be transported to the 3rd floor. | BUS |
|  | Skid Connections: The raw material inlet connections and product transfer connections will be 1” DIN 11851 Male hose connections. | BUS |
|  | Specific Requirements: Operators will wear appropriate PPE that is in line with COSHH assessment requirements, to ensure personal protection when operating/cleaning the system. Equipment layout must ensure sufficient space is maintained, to allow operator access whilst wearing PPE. | BUS |
|  | Specific Requirements: Where large items of equipment are removed for cleaning or inspection (including maintenance inspections), lifting lugs must be provided, and space available to facilitate this activity. | BUS |
|  | Specific Requirements: Where large items of equipment are removed for cleaning or inspection (including maintenance inspections), lifting beams or davit arms must be provided to facilitate this activity. | BUS |
|  | Specific limitations: Equipment installed as part of the skid must have an ingress protection of IP55 as a minimum, any deviation from this standard must be agreed. | BUS |

# Installation Requirements

| Req. No. | Requirement | Category |
| --- | --- | --- |
|  | Layout / Location Requirements: The continuous hydrogenation skid will be located in the R23 process module. R23-EH01 will be decommissioned and removed as part of this project to provide adequate space for the new skid. | BUS |
|  | Layout / Location Requirements: The TCU’s will be located in the 3rd floor plant room area, currently occupied by the caged area for storing lifting equipment. The secure store for lifting equipment will be relocated as part of this project. | BUS |
|  | Layout / Location Requirements: The continuous hydrogenation system control panel will be located in the R23 control room on the 3rd floor of the Pilot Plant. | BUS |
|  | Layout / Location Requirements: The continuous hydrogenation process vent will tie into the existing R23 module process vent, downstream of R23-FA5100 to minimise any risk of R24 vent materials back-feeding into R23. | EHS |
|  | Operating Environment Requirements: Additional extract grills will be required to enable extract air volumes to be maintained following removal of R23-EH01. No change to overall air change rates in R23 module are anticipated due to this project. | EHS |
|  | Operating Environment Requirements: The continuous hydrogenation system will be installed in an ATEX Zone 2 IIC T4 classified area. | EHS |
|  | Operating Environment Requirements: Penetrations into R23 process module must not compromise the integrity of the existing ‘Damage Limiting Construction’. | EHS |
|  | Operating Environment Requirements: Noise level generation from the continuous hydrogenation system shall not exceed 80 dB(A) for an 8-hour period. | EHS |
|  | Operating Environment Requirements Available -and/or- Required utilities: 0.8barg nitrogen and 6barg compressed air are available in the R23 module. | BUS |
|  | Operating Environment Requirements Available -and/or- Required utilities: Power (3 phase 415V) and 6barg instrument air are available in the plant room/control room area. | BUS |
|  | Specific Requirements: An earth lug will be provided on the skid framework, and a positive earth connection will be provided to prevent hazardous charges accumulating on the unit. | EHS |
|  | Other Requirements: Overpressure relief will be considered for this project. | EHS |

# Equipment and Computerised System Interfaces

| Req. No. | Requirement | Category |
| --- | --- | --- |
|  | User Interfaces: The continuous hydrogenation system will be controlled via a standalone Siemens S7 PLC with Human/Machine Interfaces (HMI) located in both the R23 process module and R23 hydrogenator control room. | BUS |
|  | Interfaces to Other Systems: The PLC will interface with the Pilot Plant DeltaV PCS via hardwired control signals, for process control interfaces. | EHS, BUS |
|  | Interfaces to Other Systems: The PLC will interface with the Pilot Plant DeltaV PCS via a communications link, for data logging and remote monitoring of equipment status. | BUS |
|  | Interfaces to Other Systems: The PLC will interface with the TCU’s via communication links for setting the TCU temperature control setpoint, data logging and remote monitoring of equipment status. | BUS |
|  | Interfaces to Other Systems: An independent, hardwired ‘Crash Cool’ process stop circuit (activated from a push button mounted on the skid) will over-ride TCU temperature control and provide emergency cooling to the continuous hydrogenator pre-heater and reactors. | EHS |
|  | Equipment Interfaces: The feed tank, product collection tank and vent headers will interface with the pilot plant 0.8barg nitrogen supply for inertion purposes. The high-pressure system must NOT be connected directly, or indirectly, to the 0.8barg nitrogen service. | EHS |
|  | Equipment Interfaces: The motive force for the hydrogen gas booster will be compressed air. The hydrogen system must not be connected directly, or indirectly, to the 6barg compressed air service. | EHS |
|  | Equipment Interfaces: The R23 batch hydrogenator and R24 continuous hydrogenator share a common emergency catch tank and process vent discharge point. The design of R24 will allow for either R23 and R24 to operate in tandem or R23 or R24 to operate independently. | BUS |
|  | Trips Interlocks and Alarms: The Pilot Plant DeltaV PCS monitors the facilities fire alarm status, HVAC status, R23 Overpressure relief status and R23 hydrogen leak detection system. If any of these status signals are NOT healthy, the R24 continuous hydrogenation will automatically transition to a safe HOLD state. | EHS |

# Computerised System Functional Requirements

This system contains computerised functions as determined by the completed SOP-VAL-002 GMP Impact/Risk Assessment.

| Req. No. | Requirement | Category |
| --- | --- | --- |
|  | Computer Functions: The continuous hydrogenator PLC will provide sequences to pressure test the system, inert the system with nitrogen prior to the introduction of flammables, purge the system with hydrogen and establish pressure control, start feeding raw materials to the reactor system, stop liquid flow to the reactors, depressurise the reactor system from hydrogen and purge with nitrogen, empty the system of liquid. Cleaning of the system will be sequenced via the PLC. | BUS, EHS |
|  | Computer Functions: User access to the PLC will be via access control with 3 user accounts pre-configured to enable user access for ‘Administrator’, ‘Engineer’ and ‘Operator’ access levels. User access to the PLC will be via a password. | BUS, EHS |
|  | Computer Functions: The continuous hydrogenator PLC will provide ‘Operators’ access to run sequences and the ability to set liquid feed rates, hydrogen flow rates, TCU temperature setpoints and system pressure via data entry points. | BUS |
|  | Computer Functions: The continuous hydrogenator PLC will provide ‘Engineers’ access to all ‘Operator’ functions and the ability to manually drive valves and carry out routine maintenance inspection, testing and basic configuration of instrumentation. | BUS |
|  | Computer Functions: PLC functionality will be detailed in a separate Functional Design Specification, Hardware Design Specification and Software Design Specification. | BUS |
|  | Computer Functions: The Pilot Plant DeltaV PCS will provide monitoring of the R23 catch tank/emergency venting system, R23 HVAC system, R23 hydrogen leak detection system and enable (if safe to do so) the hydrogen service to R24. Existing PCS documentation will be updated to include the additional I/O and the sequencing required to carry out this function. | EHS |
|  | Computer Functions: The Pilot Plant DeltaV SIS will provide high-integrity hydrogen leak detection in the R23 process module and the hydrogen service to R23 module will be disabled via safety instrumented functions if a significant leak is detected (>10% LEL). | EHS |
|  | Required Action(s) on Failure: The continuous hydrogenator PLC control system must transition to a safe state on detection of a critical fault. | EHS |

# Computerised System Data Requirements

| Req. No. | Requirement | Category |
| --- | --- | --- |
|  | Computerised System Data Types, Ranges and Limits: Existing DeltaV PCS schedules will be updated to define data types, ranges, and limits for new I/O. | BUS |
|  | Computerised System Data Types, Ranges and Limits: Continuous hydrogenator PLC schedules will be developed to define data types, ranges, and limits for new PLC I/O. | BUS |
|  | Computerised System Data Types, Ranges and Limits: Communications between the PLC and TCU’s will be via Modbus/TCP. | BUS |
|  | Computerised System Data Types, Ranges and Limits: Communications between the PLC and the Pilot Plant DeltaV PCS will be via Profibus. | BUS |
|  | Access requirements: User administration for the PLC should be configurable from the local control system HMI. | BUS |

# Data Integrity Gap Analysis Requirements

The continuous hydrogenation equipment will interface with the existing validated PCS system which is compliant with 21 CFR Part 11 for electronic records.  Electronic signatures are not used by the PCS system. The new equipment will also not be producing electronic records and therefore this section does not apply.

# Asymchem Sandwich Documentation Requirements

| Req. No. | Requirement | Category |
| --- | --- | --- |
|  | The suggested Asymchem Sandwich documentation deliverables are given in the QF11053 GMP Impact and Risk Assessment Checklist form. | GMP |
|  | B902 engineering documentation will be updated to detail the equipment installed as part of this project. As a minimum P&IDs will be prepared and valve, line, equipment and instrument schedules will be updated. | BUS |

# Supplier Documentation Requirements

| Req. No. | Requirement | Category |
| --- | --- | --- |
|  | All documentation must be in English. | BUS |
|  | Suppliers will provide an O&M manual for all equipment/packages supplied for use with the continuous hydrogenation system. | BUS |
|  | A fully indexed version of the Operation & Maintenance (O&M) Manual and an electronic copy of the manual must be supplied to Asymchem Sandwich. | BUS |
|  | The O&M Manual must include: Parts list with recommended spares identified. | BUS |
|  | Material certificates, according to EN 10204 type 3.1, for metallic product contact materials will be provided. | GMP |
|  | Certificates of conformity for product contact elastomers and seals will be provided. | GMP |
|  | UKCA/UKEX or CE/ATEX certificates will be provided where applicable. | EHS |
|  | A software design specification will be required to detail the software design. This will be required following DQ. | GMP |
|  | A list is to be provided of hardware items that include serial numbers and versions and firmware versions when built. | BUS |
|  | Back-ups of all software updates are to be provided. All media must clearly identify the computerised system and/or equipment, and the software name, release version and date for each application. | BUS |
|  | The supplier will provide a ‘Software Release Certificate’ on completion of system testing, approved by a responsible person from the supplier (e.g. Head of Quality Assurance). This document typically includes a list of key lifecycle documents issued by the supplier during the development and testing of the software. | BUS |
|  | A revision history of the software from completion of development and testing will be supplied. This will normally be the version tested at FAT and will show any subsequent changes until completion of site testing and qualification. All revisions must be clearly identified and linked to an approved software change control. | GMP |

# System Support and Training Requirements

| Req. No. | Requirement | Category |
| --- | --- | --- |
|  | A Standard Operating Procedure (SOP) or Standard Operating Instruction (SOI) shall be generated to cover the safe operation of the system. | GMP, EHS |
|  | A Training Module shall be generated to ensure that appropriate training is available to personnel involved in the operation of the system for completion prior to their operation of the system. | GMP, EHS |
|  | On site training will be provided for operators and maintenance engineers, including any relevant training certification. | GMP, EHS |

# Document History

| Version | Date | Author | Reason |
| --- | --- | --- | --- |
| 1 | 13-Apr-2026 | Keith Jefferies | New document |