Risk Register

| Operation/ Equipment/ Device | Description | Risk | Response | Risk Acceptable |
| --- | --- | --- | --- | --- |
| Engineering Design | Design Temperature | Vessel could fail during relief event. Equipment design temperature is specified for process requirements and does not consider elevated temperatures during relief events when short term temperature increase has the potential to reach 300°C. |  |  |
|  | Design Requirements | L:G ratio for plant is not clearly defined. Equipment design could result in very limited operating ranges and incorrect assumptions in plant capability and relief design. |  |  |
|  | Design Requirements | Catalyst specification is not available. Design may not be suitable for some catalysts. Dusty catalysts could cause blockages. | Design philosophy is based on catalyst beads/pellets which are dust free, non-flammable and have exposure levels requiring OEB 3 or lower containment. Note: OEB 3 containment is a deviation from the URS and requires acceptance by QA & EHS. |  |
|  | Access | Layouts are not available to assess whether access to equipment is acceptable for operations and maintenance. |  |  |
|  | Maintenance | Lifting Aids (Davit arms, lifting beams, etc) have not been considered in design with potential for injury to maintenance personnel removing heavy equipment for servicing/repair/ replacement. |  |  |
|  | Lighting | Layouts are not available to assess whether lighting for routine operations and maintenance activities is acceptable. |  |  |
| Continuous reactor build | System leak detection | Pressure testing of high-pressure system will be hydraulically tested. Hydraulic testing demonstrates mechanical strength of the pressure system but does not demonstrate integrity for a hydrogen system. | Helium testing is not routinely carried out in China. |  |
|  | Line sizing/specification | Vents/headers incorrectly sized/specified resulting in high back pressure and safety/operability issues e.g. 2-phase flow in emergency vent relief header. |  |  |
|  | Insulation | Insulation requirements not specified - could result in equipment/pipe layout not allowing space for insulation to be installed. |  |  |
|  | Modular design | Modular design could introduce significant increase in site construction activities e.g. mechanical and EC&I installation and subsequent testing. |  |  |
|  |  | Skid assembled on site could result in CE/UKCA Responsible Person/Authorised Representative in China not being able to issue Declaration of Conformity for Skid Package. |  |  |
|  | Wall penetrations | Layouts and Cable schedules not available to confirm space is available for pipe/cable penetrations to/from R23 Module. |  |  |
| Catalyst Charging | Catalyst charge point | Inlet pipe diameter is small and blockages may occur. | Inlet connection maximised (DN32 for CR01 and DN25 for CR02/CR03). Material is deemed to be free flowing beads or pellets with no risk of blocking inlet pipe. |  |
|  |  | Catalyst physical property requirements (size, shape, packing, etc.) are not defined with the potential for incompatible catalyst to be charged to the reactors. |  |  |
|  |  | Packing of catalyst is not controlled with the risk of reactants/gas channelling in the continuous reactor column, and the potential for incomplete reaction or impurities forming. |  |  |
|  |  | Contained charge system design not available to assess risks and/or design limitation. |  |  |
|  |  | Manual handling requirements for catalyst charging not assessed. |  |  |
|  |  | Access to charge point and support of charge container/bag not designed (Access steps/integrated support shelf). |  |  |
| Reactor Feed | 20-micron catalyst retaining screen | Screen blockages prevent / limit flow of gas/reaction mix to continuous reactors. | China Operations have reported blockages occur in local plant and screen has been removed and discarded. |  |
|  | Polishing filter (FTR9832) | Filter blockages prevent / limit flow of gas/reaction mix through continuous reactors. Pressure increases in reactor system, with potential for undesirable reactions to occur, until reactor high pressure trip is activated. Unable to depressurise system to change filter. |  |  |
| CR01 – CR03 reactors | Temperature control | Jacket temperature control is configured to control jacket temperature only. Reactor internal temperatures are monitored, but no control, alarms or interlocks are configured for process control. | Scale up provides large operating envelopes for process temperature requirements and additional control, alarms and interlocks are not required. |  |
| System Pressure monitoring | Differential pressure monitoring | Accuracy of static pressure measurements may not enable differential pressures to be calculated with any degree of accuracy or certainty. | China operations have reported blockages occur in local plant and screen has been removed and discarded. |  |
| Over pressure protection | Relief system design | Pressure systems not identified and relief Studies not available. Identification of worst-case relief scenario not identified, and relief streams may be incorrectly categorised. Details of maximum reaction exotherm for design purposes not available for input into PSR reviews. |  |  |
| Separator | Vessel design | Complete degassing of product stream not achieved with the potential for hydrogen carry over into BT04 and product transfer containers. |  |  |
|  |  | Cleaning of separator not fully understood but current concept relies on vessel free draining to BT04. Current spray ball sizing is not consistent with this approach and may require significant rework to process piping and/or equipment layout. |  |  |
|  |  | Cleaning of LIT003 housing does not appear to be considered in design. |  |  |
|  |  | P&ID does not clearly show nozzle locations and could lead to mal-ops/misunderstanding of the design intent, by the Operations team. |  |  |
| Sampling | Sampler design | When depressurising the sample collection chamber, high gas or vapour vent rates could result in a bubbly, or churn-turbulent flow regime, and sample will discharge to vent line as two-phase flow. |  |  |
|  |  | With significantly more gas being present in the process stream. What will prevent overpressure of sample bottle and low-pressure nitrogen pipework. |  |  |
| Catalyst Discharging | Catalyst Discharge Plug valve | Outlet pipe diameter is small and blockages may occur. | Outlet connection maximised (DN32 for CR01 and DN25 for CR02/CR03). Material is deemed to be free flowing beads or pellets with no risk of blocking outlet pipe. |  |
|  |  | Contained discharge system design not available. Layout design may not provide sufficient headroom for contained discharge system. |  |  |
|  |  | Complete removal of catalyst during discharging operations is unlikely to remove 100% of the material, with the risk of cross contamination of catalyst material between campaigns. |  |  |
| Cleaning | CIP Design | CIP design incomplete. Initial spray ball sizing is inconsistent with process line sizing approach and may require significant rework to process piping and/or equipment layout. |  |  |
|  |  | Cleaning carried out at nominally atmospheric pressure with the risk of material being trapped between flanges, which tend to separate slightly during high pressure operation, and could lead to cross contamination between campaigns. |  |  |
|  | Validation/Verification | Cleaning validation/verification philosophy not available. QA may not accept that the continuous reactor achieves minimum cleaning standards. |  |  |
|  | Layout | Layouts are not available to evaluate the impact of bypasses, dead legs, drains, vents, etc. |  |  |
|  | Layout | Layouts are not available to evaluate access to equipment for visual inspections, which may impact the effectiveness of critical inspections. |  |  |
|  | Layout | Equipment design/layout does not allow for routine disassembly and manual cleaning. Equipment turnaround times and operating costs would be adversely impacted if CIP system cannot be validated. |  |  |
| FT001 | Liquid Stream Flow Sensor for flow control loop FIC001. | FT001 is a Coriolis mass flow meter installed downstream of the metering pump MP01. Pulsed flow from the pump will impact on the flow measurement and accurate flow control may not be possible. | China Team have previously used this configuration and not experienced flow control issues. No further action required. |  |
|  | MP01 metering pump | Maximum flow may exceed separator design limitations and carry hydrogen over into BT04 / product collection containers. |  |  |
|  |  | Pump turndown does not meet URS minimum flow requirements. |  |  |
| FIC003 | Hydrogen gas flow controller | Flow controller does not support tight shut-off and system pressure increase during hold periods. | Additional shut-off valve to be installed downstream of the controller to provide tight shut-off. |  |
|  |  | Maximum flow through controller may exceed separator design limitations and carry liquid into the vent line. | High level interlock installed in the separator to stop gas supply and close PIC028 shut-off valve (CV08) |  |
|  |  | Time required for pressure testing the system and nitrogen inertion may be excessively high. | Pressure test/inertion times to be evaluated during FIC003 selection. Manual vent installed on BT03 to reduce depressurisation times via FIC003. |  |
|  |  | Differential pressure across the controller exceeds manufacturers recommendations and controller may not function correctly. | China Team reviewed with manufacturer and accuracy of flow control may be compromised but this is not deemed significant and no further action is required. |  |
| LIC003 | SE01 Separator Level Indicator | Cleaning/decontamination of housing is not possible. Residual product in housing could lead to cross contamination between campaigns. | Design relies on dissolution of residual material. 100% flood fill of housing is not possible, therefore any residual trace contamination in the housing is deemed to be small and will not significantly impact cleaning verification. |  |
| FIC004 | Separator outlet flow / level control | Differential pressure across the controller exceeds manufacturers recommendations and controller capability. |  |  |
| FIC005 | BT01 nitrogen sweep flow controller | Differential pressure across the controller is less than manufacturers recommendations and controller may not function correctly. | China Team reviewed with manufacturer and accuracy of flow control may be compromised but this is not deemed significant and no further action is required. |  |
| FIC007 | BT04 nitrogen sweep flow controller | Differential pressure across the controller is less than manufacturers recommendations and controller may not function correctly. | China Team reviewed with manufacturer and accuracy of flow control may be compromised but this is not deemed significant and no further action is required. |  |
| PIC028 | System back pressure controller | The back pressure controller does not support tight shut-off during hold periods and system pressure will not be maintained. | Additional shut-off valve to be installed downstream of the controller |  |
|  |  | Time required for depressurising the system following pressure testing and nitrogen inertion may be excessively high. | Manual bypass installed around PIC028 controller. |  |
|  |  | Differential pressure across the controller exceeds manufacturers recommendations and controller may not function correctly. |  |  |
| PIC030 | Separator liquid outlet pressure reducer | Is this confirmed as a Bronkhorst pressure controller, or is a mechanical regulator still an option? Does this provide the required independence assumed in the relevant SIL/Risk assessments. |  |  |
| PIC049 | BT04 back pressure controller | Differential pressure across the controller is less than manufacturers recommendations and controller may not function correctly. | China Team reviewed with manufacturer and accuracy of flow control may be compromised but this is not deemed significant and no further action is required. |  |
| Automation | Automation Build | Philosophy and Design documentation not available to assess risks and limitations. |  |  |
| Documentation | Technical File | Technical File not available to assess risks and limitations. |  |  |
|  | O&M Manual | O&M manual not available to assess risks and limitations. |  |  |
|  | Emissions Calculations | Calculations to demonstrate expected release of Class A and Class B VOC’s to atmosphere are not available. These are required to demonstrate compliance with EA Permit limits. |  |  |
| Regulatory compliance | CE Functional Safety DSEAR / Atex PSSR COSHH PUWER (Access, manual handling) Working at Height (Access Steps/Support Shelf) LOLER (Davit arms) Lighting Occupational Hygiene EA Permit | Regulatory non-conformance would prevent beneficial use of equipment in the UK. |  |  |